Request for Quotation (RFQ) for

Implementation of an Integrated Enterprise GIS and 3D Digital Twin Platform for Geo-Analytics, Property Tax Enhancement, Asset Management, and Urban Governance for Mysuru City

RFQ No.: DMA/2026-27/IND6251

Issued By:

MYSURU CITY CORPORATION (MCC)

Next To Banumaiah College, Sayyaji Rao Rd, Agrahara, Chamrajpura, Mysuru - 570024.

- Table of Contents

1	Introduction	3

2	Scope of Work	4

2.1	Part A	4

2.1.1	Establishment of Continuously Operating Reference Stations (CORS)	4

2.1.2	Door-To-Door Survey of Properties	4

2.1.3	Development of Web-Based Property Tax Management System	5

2.1.4	Development of Web-Based Asset Management System	5

2.1.5	Supply of Web-Based Location Intelligence Platform	6

2.1.6	Security Audit and Certification	6

2.1.7	Training and Orientation	7

2.1.8	Ground Truthing	7

2.2	Part B	7

2.2.1	Supply of 3D Digital Twin of City with 2D/3D Data Layers	7

2.2.2	Supply of Unified 3D Web-GIS platform	10

2.3	Part C – Annual Maintenance	12

3	Project Deliverables	13

4	Bill of Quantities	14

5	Terms and Conditions	17

6	Appendix	19

Annexure-A: List of Layers to be included in the 2D Base Map	19

Annexure-B: List of Layers to be included in the LULC Map	40

# Introduction

The Mysuru City Corporation (MCC) is responsible for urban governance across a rapidly evolving municipal jurisdiction that requires accurate, integrated and up-to-date spatial information to support planning, service delivery and revenue administration. This Request for Qualification (RFQ) is being issued by MCC for the implementation of an Integrated Enterprise GIS and 3D Digital Twin Platform for Geo-Analytics, Property Tax Enhancement, Asset Management and Urban Governance, with the aim of enabling integrated and accurate spatial data management, enhancing property tax assessment, strengthening utility asset management and supporting efficient urban governance in Mysuru.

The Area of Interest (AoI) for the project covers the entire jurisdiction of MCC, encompassing all municipal wards, planned layouts and peri-urban expansion zones within city limits.

The proposed implementation will be based on advanced geospatial data acquisition and integration, including aerial LiDAR and photogrammetry surveys and mobile mapping surveys for LiDAR and 360° panoramic imagery capture covering approximately 86 sq. km. and door-to-door property surveys covering approximately 2,50,000 properties. In addition, two (2) Continuously Operating Reference Stations (CORS) shall be established to ensure high-precision geospatial referencing and survey control. These datasets shall collectively form the foundation for the development of a high-resolution 3D digital twin of the city.

The platform will enable seamless integration of spatial and non-spatial datasets, providing a unified environment for visualization, analysis, monitoring and decision-making across municipal departments. It will support property tax assessment and enhancement through improved spatial accuracy and data reconciliation, urban planning, operational monitoring and change detection.

Through this RFQ, MCC seeks to identify qualified agencies with demonstrated capability in large-scale enterprise GIS, 3D city modelling, spatial data integration and municipal system development to quote rates for a scalable and interoperable solution.

The engagement is intended to establish a robust geospatial foundation for Mysuru that strengthens municipal revenue systems, improves urban planning capabilities, enhances inter-departmental coordination and enables data-driven and sustainable urban governance.

# Scope of Work

## Part A

### Establishment of Continuously Operating Reference Stations (CORS)

- The Bidder shall review the existing Survey of India (SOI) CORS network and plan for the establishment of two new CORS stations within MCC jurisdiction.

- Suitable MCC-owned or other secure sites shall be identified by the Bidder, for the installation of the CORS stations, ensuring optimal coverage and geodetic accuracy across the AoI.

- The Bidder shall procure, install, commission and test the CORS hardware and associated civil structures, including control centers, air-conditioning (if required), UPS, internet connectivity and all other required materials.

- The Bidder shall conduct functional testing of the CORS network, using standard testing procedures and document the results in a formal report for MCC review.

- The Bidder shall develop a secure web-based interface or service for accessing the CORS network, incorporating secure access controls.

- Upon successful installation, commissioning and testing, the Bidder shall coordinate with SOI to obtain certification confirming that the CORS network is fully functional, accurate and suitable for GIS and surveying applications.

- Any subsequent additions or modifications to the CORS stations shall be re-certified by SOI.

### Door-To-Door Survey of Properties

- The Bidder shall conduct door-to-door survey of all properties within the AoI using a mobile application, to capture “as-is” property details.

- The survey shall capture, at minimum, the following:

  - Property Information

- Property/Tax ID, Present Name, Complete Address (Pin Code, Street, Floor)

- Property Age, Height, Number of floors

- Property Type (Ex. Residential / Commercial / Mixed- Residential + Commercial)

- Ownership Type (Ex. Government Owned / Private)

- No. of rooms, Sub-divisions, Construction Material

  - Owner & Occupancy Details

- Owner Details, Length of Ownership, Service Type

- Occupancy status (Ex. Owner-occupied / Rented)

  - Surveyor Observations (Notes / Photographs / Videos)

- Survey Methodology & Integration

  - The survey form for data collection shall be provided by MCC.

  - Survey sequencing shall be finalized in consultation with MCC officials.

  - Property details from government records, surveys and other sources shall be gathered by the Bidder and linked to land parcel boundaries.

  - A Property Numbering System workflow shall also be defined by the Bidder for generating unique IDs for the proposed Property Tax Management System (PTMS).

  - Quality checks shall be performed during data collection by the Bidder.

  - The Bidder shall integrate the captured property data with the 3D Web-GIS and Property Tax Management systems to be developed as part of this project, enabling verification, reconciliation and analysis by the concerned officials of MCC.

### Development of Web-Based Property Tax Management System

The Bidder shall design and develop a Web-based Property Tax Management System (PTMS) for property management, assessment and tax administration. The system shall support unlimited concurrent MCC and citizen users for viewing and payment of property tax bills. The system shall also enable seamless integration with existing MCC systems and relevant line department portals.

The system shall comply with the following specifications and capabilities:

- Allows MCC officers to manage properties, update ownership records and other property attributes, assess taxes and generate property tax demands and bills.

- Supports citizen-facing services, allowing property owners to securely log in, access tax demand notices, payment history and make online payments through supported payment gateways.

- Provides interactive dashboards and analytical reports for MCC officials to monitor tax demand, collections, arrears and overall revenue performance.

- Supports configurable tax assessment rules and workflows.

- Supports secure authentication and access control mechanisms.

- Supports seamless integration with the proposed 3D Web-GIS platform, other enterprise systems and third-party applications to enable automated data exchange for property records, tax assessment, citizen payments and consolidated reporting.

### Development of Web-Based Asset Management System

The Bidder shall design and develop a Web-based Asset Management System for visualization and management of utility assets through a geospatial interface. The system shall support unlimited concurrent users and shall comply with the following specifications and capabilities:

- Provides access to the published geospatial datasets for visualization and management.

- Enables standard map interactions including zooming, panning and layer control.

- Supports distance and area measurement tools.

- Provides location search by attribute or coordinates.

- Supports proximity-based identification of features within a user-defined buffer around points, lines or polygons.

- Provides functionality to view attribute information of selected map features.

- Allows users to raise, resolve, manage, and delete issues by creating points/lines/polygons to tag feature layers

- Enables seamless integration of panoramic street view imagery

- Allows users to generate reports in Excel and/or PDF format

- Supports role-based access and user management functionalities.

### Supply of Web-Based Location Intelligence Platform

The Bidder shall supply a Web-based Location Intelligence Platform (LIP) for integrated spatial visualization and analysis to support urban governance and data-driven decision-making. The system shall support unlimited concurrent users and shall comply with the following specifications and capabilities:

- Integrates both geospatial and non-spatial datasets, including GIS layers and customer-provided or externally sourced information.

- Displays geospatial and thematic data through interactive 2D map views, enabling exploration of spatial patterns and trends.

- Provides centralized dashboards displaying current and historical data, analytical insights and alerts for informed decision-making.

- Supports dynamic exploration of spatial data with querying, filtering and buffer analysis, to identify patterns and relationships.

- Enables upload and overlay of external datasets onto existing map layers, multiple dataset management and flexible geospatial representation (points, thematic maps, charts/tables).

- Allows customizable map styles, labels and icons for improved visualization and interpretation.

- Provides automated report generation with historical trend analysis.

- Implements secure, role-based user authentication and management with audit logs.

### Security Audit and Certification

- The Bidder shall ensure that all applications developed under the scope undergo comprehensive security audit and certification.

- The audit shall include Vulnerability Assessment and Penetration Testing (VAPT) and load testing, to be conducted by a CERT-In empaneled agency.

### Training and Orientation

- The Bidder shall provide theory, hands-on, refresh session and continuous support to familiarize the MCC’s team with the use and management of the delivered data and platforms.

- The Bidder shall prepare and provide comprehensive and well-structured training materials such as presentations, user manuals and instructional videos, to facilitate effective learning.

- The Bidder shall train 2 resources of MCC of the entire project cycle to be able to operate independently beyond AMC period.

### Ground Truthing

- Ground validation shall be carried out for 5% of the surveyed area jointly by the Bidder and MCC teams.

- Any discrepancies identified shall be corrected by the Bidder and incorporated into the final deliverables.

## Part B

### Supply of 3D Digital Twin of City with 2D/3D Data Layers

- The Bidder shall supply and integrate geospatial datasets derived from:

  - Ground Control Network (GCN)

  - Aerial photogrammetry and LiDAR surveys

  - Mobile mapping surveys with LiDAR and 360° imagery

All statutory, aviation, and operational permissions required for aerial/mobile survey execution shall be obtained by the Bidder at its own cost unless specifically stated otherwise. MCC shall provide supporting letters, NOCs, and coordination assistance wherever required.

- The data deliverables shall include:

  - Aerial & Mobile LiDAR Point Cloud

  - 360-Degree Panoramic Street View Imagery

  - Digital Elevation Model (DEM), Digital Surface Model (DSM) and Digital Terrain Model (DTM) & Contours

  - Aerial Imagery (True Orthophoto)

  - Base Map

  - Land-Use Land-Cover Map

  - 3D Mesh Model

  - 3D Vector Models (Up to LOD2)

- The datasets shall be integrated into a Unified 3D Web-GIS platform supporting visualization, spatial analysis and decision-making.

- The platform shall support unlimited concurrent users.

#### Technical Specifications of Data Layers

The 2D and 3D geospatial data layers supplied under this project shall adhere to the following technical specifications:

##### True Orthorectified Imagery

- The imagery shall be true ortho-rectified, with geometric distortions eliminated through standard photogrammetric rectification.

- Ground Sampling Distance (GSD) shall be 5 cm or better.

- Horizontal accuracy shall be ≤ 10 cm in both X and Y directions.

- Radiometric resolution shall be minimum 8 bits per band, in accordance with the selected image format.

##### LiDAR Point Cloud

- The LiDAR point cloud shall be high-density, suitable for accurate surface and object representation.

- Aerial LiDAR point cloud shall have a minimum density of 10 points per square meter (ppsm).

- Mobile LiDAR point cloud shall have a minimum density of 300 points per square meter (ppsm) at 10 m distance from the sensor.

##### Elevation Models and Contour Maps

- Digital Elevation Model (DEM), Digital Surface Model (DSM) and Digital Terrain Model (DTM) shall be derived from aerial LiDAR data and shall represent the surface and terrain accurately.

- The elevation models shall adhere to the following specifications:

- Grid interval of the DEM/DSM/DTM shall be 50 cm or better.

- Permanent water bodies like lakes, ponds, reservoirs, etc. shall be hydro-flattened, whereas linear water bodies like rivers, streams, etc. shall be hydro-enforced.

- Bridges shall be removed from the DEM and DSM. Roads or other travel ways over culverts shall remain intact in the surface model. Break lines shall be properly incorporated on the edges of rivers, water bodies and hilly areas.

- The DEM, DSM and DTM shall be corrected for different categories of vegetation, different heights of built-up areas, etc. and shall be seamless across edges.

- Contours derived from the elevation models shall adhere to the following specifications:

- Contour intervals shall be 50 cm.

- Breakline data shall be in the same CRS as the LiDAR data.

- Each breakline feature class shall have properly formatted, accurate and complete georeferenced information stored in the format’s standard file system location.

- All features shall be Z- and M-value enabled, tagged with respective contour values.

##### 2D Base Map

- The 2D base map shall be derived from ortho-rectified imagery and shall represent all relevant physical, geographic and administrative features within the AoI such as existing administrative boundaries, building footprints, property boundaries, roads, contours, natural features such as water bodies etc. to create a comprehensive GIS database.

- The resulting vector data shall be geo-referenced, accurate and classified into appropriate layers to generate a base map at a scale of 1:500, adhering to the minimum standards and specifications outlined in the AMRUT Guidelines (layer details provided in Annexure-A).

- The base map shall include relevant attributes for each feature and support integration with other geospatial layers as needed.

##### LULC Map

- Land Use Land Cover (LULC) map of the AoI shall be derived from the interpreted and digitized 2D base map features and shall represent standardized land use and land cover categories such as residential, commercial, industrial, public/semi-public, transportation, vegetation, water bodies and open spaces.

- The LULC map shall be prepared at a scale of 1:500, adhering to the minimum standards and specifications prescribed under the AMRUT Guidelines (layer details provided in Annexure-B).

##### 3D Mesh Model

- The 3D mesh model shall be high-resolution and textured, representing the urban and rural environment of the AoI.

- 3D mesh model shall adhere to the following specifications:

- The mesh model shall be generated from aerial LiDAR and/or nadir and oblique imagery.

- The model shall be seamless in nature and shall depict all visible building structures, roads, assets etc.

- The model shall have realistic view/textures.

- The model shall be corrected to the values derived from the DSM.

- Data accuracy shall be:

- Horizontal Accuracy: ± 10 cm

- Vertical Accuracy: ± 25 cm

##### 3D Vector Models

- 3D vector models shall be produced up to LOD2 using building footprints and height information derived from LiDAR and photogrammetric data.

- The 3D vector models shall adhere to the following specifications:

- The LOD2 representation shall depict the outer shell of the building including vertical exterior surfaces and simplified roof shapes.

- All relevant surface types such as ground surfaces, wall surfaces, roof surfaces, outer ceiling surfaces and outer floor surfaces, shall be represented as semantic objects.

- The 3D vector models shall be OGC compliant.

##### 360-Degree Panoramic Street View Imagery

- 360-degree panoramic street view imagery shall be seamless and suitable for geospatial integration and visual validation.

- The panoramic imagery shall adhere to the following specifications:

- Minimum overall resolution of 24 megapixels for the stitched 360-degree panoramic imagery, with a per-sensor resolution of at least 12 megapixels.

- At least 8K resolution at a minimum of 5 frames per second.

- The imagery shall offer a 360-degree horizontal field of view and a contiguous field of view of at least 135 degrees.

### Supply of Unified 3D Web-GIS platform

The Unified 3D-Web GIS platform shall be compliant with the following specifications and capabilities:

- Provides access to the published geospatial datasets for visualization, navigation and analysis, with a table of contents (TOC) for navigating published layers and the ability to seamlessly switch between 2D and 3D views, covering:

- Ortho imagery

- LiDAR point clouds

- DEM/DSM/DTM

- Contours

- 2D base map

- LULC maps

- 3D mesh and vector models

- 360° panoramic street view imagery

- Offers zooming, panning and rotation functionalities for smooth map interaction.

- Supports measurement of distances, polygonal areas, building heights and other 3D asset dimensions.

- Enables intelligent search by area/locality name or coordinates for rapid location discovery.

- Allows creation, saving and retrieval of bookmarks for specific locations or views across sessions.

- Supports marking of locations using customizable drawing tools, with options to add notes, labels or descriptions.

- Provides interactive retrieval of detailed attribute information for map features, including integrated property details and attachments by selecting building footprints.

- Enables creation of spatial buffers around points, lines or polygons to identify nearby features.

- Supports analysis for identification of assets or features impacted by proposed activities within a defined radius or area of influence.

- Provides intelligent routing to determine optimal paths from defined start to end points.

- Supports visualization of reports and dashboards.

- Implements secure access management through:

- Role-Based Access Control (RBAC) to ensure controlled access to data.

- Audit logs to track user activities for accountability.

- Supports secure integration with internal and external systems, legacy applications, historical datasets, CCTV feeds, sensors and IoT devices through standard APIs and SDKs for seamless data exchange and interoperability.

#### Development of Applications within the Unified 3D Web-GIS Platform

The Bidder shall develop the applications listed below in accordance with the functional requirements defined herein.

| S.No. | Use Case | Expected Functionality |
| --- | --- | --- |
|  | Property Tax Rationalization and Leakage Detection | - GIS-based visualization of properties with associated attributes, tax assessments and payment records. - Identification and display of discrepancies between field data and existing records including missing assessments and undervalued properties. |
|  | Development Plan Visualization & Impact Analysis | - Overlay of existing 2D master plans onto 3D GIS datasets to visualize proposed urban developments in context with the current city environment. - Support for spatial queries to assess the potential impact of proposed developments. - Detection of new building construction using spatio-temporal analysis. |
|  | License Management for Advertisement Hoardings | - Geo-tagging and 3D visualization of advertisement hoardings / billboards. - Measurement and verification of hoarding dimensions, height, area and proximity relative to regulatory norms. - Identification and reporting of violations or deviations from permissible standards. |

## Part C – Annual Maintenance

#### Change Detection for Property Data Updation

- Post Go-Live, Bidder shall procure high-resolution satellite imagery (5 cm resolution) covering the AoI once every year during the three (03) year AMC period to facilitate change detection in property coverage.

- A comparative assessment shall be conducted by the Bidder to evaluate variations between existing GIS building footprints and updated imagery-derived footprints using advanced AI/ML algorithms to identify changes/developments such as newly constructed buildings, demolished buildings and encroachments.

- An on-site project team shall be deployed by the Bidder to incorporate all verified changes into the GIS database.

#### Annual Maintenance of Applications

The Bidder shall provide technical support services for all applications for a duration of three (3) years post Go-Live.

Key activities shall include:

- Identification, diagnosis and resolution of software bugs, errors and functional issues reported by users.

- Resolution of user queries and operational issues related to application usage and functionality.

- Maintenance of records for reported issues, response timelines and resolutions.

- Periodic review of system performance and application logs for proactive identification and resolution of potential issues.

# Project Deliverables

The following deliverables shall be provided by the Bidder:

- Establishment and operationalization of two (2) Continuously Operating Reference Stations (CORS), including access interface and SOI certification

- Web-based Property Tax Management System (PTMS) with integrated door-to-door property survey data

- Web-Based Asset Management System

- Web-Based Location Intelligence Platform (LIP)

- Unified 3D Web-GIS platform along with property survey data and customized applications as specified in the RFQ.

- Aerial Imagery (True Orthophoto) (≤ 5 cm GSD)

- Aerial LiDAR Point Cloud (≥ 10 ppsm)

- Mobile LiDAR Point Cloud (≥ 300 ppsm)

- DEM / DSM / DTM (50 cm grid interval)

- Contours (≤ 50 cm intervals)

- Base Map (1:500 scale)

- Land-Use Land-Cover Map (1:500 scale)

- 3D Mesh Model (Horizontal Accuracy: ± 10 cm and Vertical Accuracy: ± 25 cm)

- LOD2 Vector Model (Buildings with simplified roof structures)

- 360-Degree Panoramic Street View Imagery (≥ 24 MP stitched)

- Post-Implementation Support for 3-years after Go-Live

- Annual maintenance of the applications and infrastructure delivered under the project scope

- Change detection maps for properties

- On-site support for property data update

The ownership of all source code, Perpetual license, Database schema, API documentation and others will be handed over to MCC.

# Bill of Quantities

| Sr. No | Activities | Unit | Quote |  |
| --- | --- | --- | --- | --- |
|  |  |  | Unit Rate | Amount in INR |
| A | Part A |  |  |  |
| 1 | Establishment of Continuously Operating Reference Stations (CORS) | Nos |  |  |
| 2 | Door-to-Door Survey of Properties | No. of properties |  |  |
| 4 | Development of Web-based Property Tax Management System | No. of properties |  |  |
| 5 | Integration of existing systems of MCC and other line department portals with Web-based Property Tax Management System | Per Sq. Km |  |  |
|  | Comprehensive audit including security Certification, VAPT from CERT-IN empanelled agency for all applications developed as per the scope of work |  |  |  |
| 6 | Web-Based Location Intelligence Platform | Per Sq. Km |  |  |
| 7 | Web-based Asset Management System | Per Sq. Km |  |  |
| B | Part B |  |  |  |
| 8.1 | Supply of 3D Digital Twin of city along with following data: a) Ground control points using DGPS/CORS. b) High-resolution ortho- imagery with a Ground Sampling Distance (GSD) of 5 cm and of resolution of 150 megapixels or higher using oblique cameras mounted on manned aircraft/drone, i.e. aerial survey with LiDAR and photogrammetric sensor c) DEM, DSM @ 50cm resolution captured from aerial LiDAR data, d) 3D Aerial LiDAR Point Cloud (min 10ppsm), e)3D Mesh Model generated from nadir and oblique imagery with realistic view/textures corrected from the values generated from DSM and data accuracy- vertical 20cm & horizontal 15cm f) LOD 2 Model: OGC complied 3D vector models using building outline with building height etc. as mentioned in the scope of work g) Base map creation, updation & digitization of all vector layers mentioned in the Scope of Work h)Base Maps (1:500 scale) as per the AMRUT Guidelines, i)Mobile LiDAR with point density of 300 ppsm for roads and 360 Degree panoramic Street View imagery data for all motorable roads with resolution of the stitched imagery of minimum 24 megapixels, with minimum resolution per sensor at least 12 megapixels. j) Creation of 3D City Digital twin model of city using Aerial LiDAR & photogrammetry survey, 360˚Panoramic Street view & Mobile LiDAR survey. | Per Sq. Km |  |  |
| 8.2 | The Unified 3D Web-GIS Platform shall include: -Visualization and option to switch between of 2D and 3D map layers and Mobile LiDAR + 360˚panaromic street view imagery -Feature to visualize building information and detect deviations in property based on building records and property survey data -Measurement tool to measure building height and other 3D assets -Perform change detection analysis for detection of new properties (Refer Section 2.2.2 for use cases). | Per Sq. Km (Unlimited users) |  |  |
| C | Part C |  |  |  |
| 9 | Satellite Data for Change Detection | Per Sq. Km |  |  |
| 10 | Annual Maintenance of Unified 3D GIS Viewer Application | Yearly |  |  |
| 11 | Annual Maintenance of Property Tax Management System | Yearly |  |  |
| 12 | Annual Maintenance of Web-based Asset Management System | Yearly |  |  |
| 13 | Annual Maintenance of Web-based Location Intelligence Platform | Yearly |  |  |
| 14 | On-site Support for Property Data Updation (2 Resources on-site) | Yearly |  |  |
| Project Extent: • Area of Interest (AoI): Approximately 86 Sq. Km. • Number of Properties for Door-to-Door Survey: Approximately 2,50,000 Properties. • CORS Stations to be established: 2 Nos. Bidders shall quote strictly on PER UNIT RATE basis against each line item above. Total values shall be derived by multiplying quoted unit rates with indicative quantities. Quantities are approximate and may vary based on actual execution. |  |  |  |  |

# Terms and Conditions

Note: The Bidder may collaborate with an OEM for any product or solution supplied as part of this project and submit their OEM eligibility documents. If the bidder itself is the OEM, then they should meet both the bidder and OEM eligibility criteria. No JV or consortium allowed.

- Bidder Eligibility

The Bidder should meet all the criteria given in this section (criteria, sub-criteria and point system for evaluation of this proposal):

- The bidder has to submit methodology and proposal for the work of Implementation of an Integrated Enterprise GIS and 3D Digital Twin Platform for Geo-Analytics, Property Tax Enhancement, Asset Management, and Urban Governance for Mysuru City.

- The bidder should be registered in India under Companies Act 1956/2013/LLP Act 2008/Society Act or as amended and should have been in operation in similar business for at least 8 years in India as on the date of submission of the RFQ. The Bidder should submit a valid copy of the Certificate of Incorporation.

- The Bidder should have minimum average annual turnover of INR 20 Crores from Remote Sensing/Photogrammetry/GIS services, Aerial Aircraft, Mobile LiDAR survey or IT/ITES services for any two years in the last five (5) financial years ending 31st March of the previous financial year i.e. FY 2021-22, 2022-23, FY 2023-24, FY 2024-25, FY 2025-26 (standalone income in India). Credentials of group companies/parent companies/subsidiaries shall not be allowed.

- The bidder must possess valid ISO 9001:2015 and ISO 27001:2013 from a recognized institute.

- The bidder should have a valid PAN and GST registration certificate.

- The bidder should have successfully performed door-to-door survey/attribute collection or validation in a single project for at least 75,000 units integrated with an existing/new GIS platform/application for any State/Central Government/PSU/Semi-Government/ULB in the last seven (7) years. The bidder shall submit relevant Work Order/PO along with a Completion Certificate issued by a competent authority not below the rank of Executive Engineer/Head of Department or equivalent

- OEM Eligibility Criteria

- The OEM shall have similar work experience of supply of 3D Digital twin data for a minimum of 100 sq. km for any city in India using aerial LiDAR survey through manned aircraft, integrated with Mobile LiDAR and 360-degree panoramic images along with 3D city models in the last seven (7) years, with a minimum contract value of INR 10 Crores. The vendor shall submit the relevant Work Order/PO along with a Completion Certificate issued by a competent authority not below the rank of Executive Engineer/Head of Department or equivalent.

- The OEM shall have experience in 3D Web GIS application development and maintenance support in the last seven (7) years with a minimum project value of INR 5 Crores for any Central/State/Semi-Government Company in India. Relevant Work Order/PO along with Completion Certificate must be submitted.

- The proposed OEM product should be available on the GeM Portal and should have a valid Trade Registration Certificate or any equivalent certificate issued by a competent authority of the Government of India/State/PSU.

- The proposed OEM must possess a valid CMMI Level 3 certification from a recognized institute.

- Quoted Price

- The quoted price shall be provided for each item as described above.

- The quoted price should be inclusive of all applicable costs; however, GST shall be paid extra as applicable.

- The rates quoted by the bidder shall remain fixed for a period of one (1) year from the date of opening of the quotation and shall not be subject to any escalation.

- The rates shall be inclusive of transportation and installation charges.

- The bidder shall cooperate and coordinate with department for necessary approvals.

- Submission of Quotations

- The bidder is advised to visit the site of work at their own expense and obtain all necessary information required for preparing the quotation.

- Each bidder shall submit only one quotation.

- Bidders are requested to submit their offer latest by 17:00 hrs.

- Date of submission: 11/05/2026

# Appendix

## Annexure-A: List of Layers to be included in the 2D Base Map

| Layer Name | Class | Sub Class |
| --- | --- | --- |
| Road Polygon | Road Polygon | National Highway |
| Road Polygon | Road Polygon | State Highway |
| Road Polygon | Road Polygon | Major District Road |
| Road Polygon | Road Polygon | Other District Road |
| Road Polygon | Road Polygon | Expressway |
| Road Polygon | Road Polygon | Bypass |
| Road Polygon | Road Polygon | Ring Road |
| Road Polygon | Road Polygon | Service Road |
| Road Polygon | Road Polygon | Major City Road (Width 10.5 m and more) |
| Road Polygon | Road Polygon | Minor City Road (Width less than 10.5 m) |
| Road Polygon | Road Polygon | BRTS |
| Road Polygon | Road Polygon | Cycle Track |
| Road Polygon | Road Polygon | Proposed Road |
| Road Polygon | Road Polygon | DP Road |
| Road Polygon | Road Polygon | HCMTR |
| Water Bodies Line | Water Bodies Line | Stream |
| Water Bodies Polygon | Water Bodies Polygon | Stream |
| Water Bodies Polygon | Water Bodies Polygon | Lake |
| Land Use/Landcover | Residential Zone | Residential Area |
| Land Use/Landcover | Slum | Notified Slum |
| Land Use/Landcover | Slum | Non-Notified Slum |
| Land Use/Landcover | Vacant Land | Vacant Land |
| Land Use/Landcover | Green Areas | Notified Forest |
| Land Use/Landcover | Green Areas | Tree Clad Area |
| Land Use/Landcover | Agricultural Land / Zone | Fallow land |
| Land Use/Landcover | Specific Land Use | Hill |
| Land Use/Landcover | Others | Dam |
| Land Use/Landcover | Others | Barrage |
| Cell Tower, Wi-Fi Hotspot & Public Telephone Booth | Cell Tower, Wi-Fi Hotspot & Public Telephone Booth | Cell Tower |
| Tree | Tree | Tree |
| Building | Residential | Residential Area |
| Administrative Boundary | Administrative Boundary | Forest Boundary |
| Road Line | Road Line | National Highway |
| Road Line | Road Line | State Highway |
| Road Line | Road Line | Major District Road |
| Road Line | Road Line | Other District Road |
| Road Line | Road Line | Expressway |
| Road Line | Road Line | Bypass |
| Road Line | Road Line | Ring Road |
| Road Line | Road Line | Service Road |
| Road Line | Road Line | Major City Road (Width 10.5 m and more) |
| Road Line | Road Line | Minor City Road (Width less than 10.5 m) |
| Road Line | Road Line | BRTS |
| Road Line | Road Line | Cycle Track |
| Road Line | Road Line | Foot Path |
| Road Line | Road Line | Cart Track |
| Road Line | Road Line | Ropeway |
| Road Line | Road Line | Carriageway |
| Road Line | Road Line | Right of way |
| Road Line | Road Line | Proposed Road |
| Road Line | Road Line | DP Road |
| Road Line | Road Line | HCMTR |
| Rail Line | Rail Line | Broad Gauge |
| Rail Line | Rail Line | Narrow Gauge |
| Rail Line | Rail Line | Meter Gauge |
| Rail Line | Rail Line | Metro/MRTS Line |
| Rail Line | Rail Line | Metro/MRTS Line |
| Rail Polygon | Rail Polygon | Metro Station |
| Rail Polygon | Rail Polygon | TOD Corridor along Railway |
| Rail Polygon | Rail Polygon | MRTS |
| Bridges | Bridges | Culvert |
| Bridges | Bridges | Tunnel |
| Bridges | Bridges | Bridge across River |
| Bridges | Bridges | Over Bridge |
| Bridges | Bridges | Under Pass |
| Bridges | Bridges | Road Bridge across Rail |
| Bridges | Bridges | Subway |
| Bridges | Bridges | Foot over Bridge |
| Bridges | Bridges | Rope bridge |
| Flyovers | Flyovers | Flyover |
| Water Bodies Line | Water Bodies Line | Canal |
| Water Bodies Line | Water Bodies Line | Drain |
| Water Bodies Polygon | Water Bodies Polygon | River |
| Water Bodies Polygon | Water Bodies Polygon | Canal |
| Water Bodies Polygon | Water Bodies Polygon | Drain |
| Water Bodies Polygon | Water Bodies Polygon | Ponds |
| Water Bodies Polygon | Water Bodies Polygon | Tank |
| Water Bodies Polygon | Water Bodies Polygon | Island (River) |
| Water Bodies Polygon | Water Bodies Polygon | Island (Lake) |
| Water Bodies Polygon | Water Bodies Polygon | Reservoir |
| Water Bodies Polygon | Water Bodies Polygon | Back Water |
| Water Bodies Polygon | Water Bodies Polygon | Sea |
| Land Use/Landcover | Residential Zone | Township (ITP) |
| Land Use/Landcover | Residential Zone | Public Housing Scheme |
| Land Use/Landcover | Residential Zone | Earlier Sanctioned Layout |
| Land Use/Landcover | Residential Commercial Zone | Retail |
| Land Use/Landcover | Residential Commercial Zone | Wholesale |
| Land Use/Landcover | Residential Commercial Zone | General Business |
| Land Use/Landcover | Residential Commercial Zone | Hotel |
| Land Use/Landcover | Residential Commercial Zone | Lodge |
| Land Use/Landcover | Residential Commercial Zone | Restaurant |
| Land Use/Landcover | Residential Commercial Zone | Shopping Centre |
| Land Use/Landcover | Residential Commercial Zone | Shopping Mall |
| Land Use/Landcover | Residential Commercial Zone | Multiplex |
| Land Use/Landcover | Residential Commercial Zone | Cinema |
| Land Use/Landcover | Residential Commercial Zone | Function Hall |
| Land Use/Landcover | Residential Commercial Zone | Marriage Garden |
| Land Use/Landcover | Residential Commercial Zone | Resort |
| Land Use/Landcover | Residential Commercial Zone | Petrol Pump |
| Land Use/Landcover | Residential Commercial Zone | LPG filling station |
| Land Use/Landcover | Residential Commercial Zone | Informal Shop |
| Land Use/Landcover | Residential Commercial Zone | Hostel |
| Land Use/Landcover | Residential Commercial Zone | Market (Daily & Weekly) |
| Land Use/Landcover | Residential Commercial Zone | Mandi |
| Land Use/Landcover | Residential Commercial Zone | Private Office |
| Land Use/Landcover | Residential Commercial Zone | Banks |
| Land Use/Landcover | Residential Commercial Zone | Tourist Facility Centre |
| Land Use/Landcover | Residential Commercial Zone | Earlier Sanctioned Layout |
| Land Use/Landcover | Industrial | Manufacturing |
| Land Use/Landcover | Industrial | Service |
| Land Use/Landcover | Industrial | Chemical |
| Land Use/Landcover | Industrial | Pharmaceutical |
| Land Use/Landcover | Industrial | Textile |
| Land Use/Landcover | Industrial | IT Parks |
| Land Use/Landcover | Industrial | Industrial Estate |
| Land Use/Landcover | Industrial | SEZ |
| Land Use/Landcover | Industrial | Agro Based & Food Processing |
| Land Use/Landcover | Industrial | Obnoxious |
| Land Use/Landcover | Industrial | Cottage and Household |
| Land Use/Landcover | Industrial | Other Industries |
| Land Use/Landcover | Industrial | Warehouse |
| Land Use/Landcover | Industrial | Storage Godown |
| Land Use/Landcover | Industrial | Earlier Sanctioned Layout |
| Land Use/Landcover | Mixed | Residential & Commercial |
| Land Use/Landcover | Mixed | Residential & Household Industry |
| Land Use/Landcover | Mixed | Residential & Educational |
| Land Use/Landcover | Mixed | Residential & Health Services |
| Land Use/Landcover | Mixed | Commercial & Industrial |
| Land Use/Landcover | Mixed | Commercial & Health Services |
| Land Use/Landcover | Mixed | Commercial & Educational |
| Land Use/Landcover | Mixed | Commercial & Recreational |
| Land Use/Landcover | Mixed | Residential & Commercial & Institutional |
| Land Use/Landcover | Mixed | Residential & Handloom Industry |
| Land Use/Landcover | Mixed | Other |
| Land Use/Landcover | Mixed | Earlier Sanctioned Layout |
| Land Use/Landcover | Educational | School |
| Land Use/Landcover | Educational | College |
| Land Use/Landcover | Educational | University |
| Land Use/Landcover | Educational | Vocational Institute |
| Land Use/Landcover | Educational | ITI |
| Land Use/Landcover | Educational | Anganwari |
| Land Use/Landcover | Educational | Training Institute |
| Land Use/Landcover | Educational | Earlier Sanctioned Layout |
| Land Use/Landcover | Health Services | Govt. Hospital |
| Land Use/Landcover | Health Services | Rural Hospital |
| Land Use/Landcover | Health Services | Municipal Hospital |
| Land Use/Landcover | Health Services | Private Hospital |
| Land Use/Landcover | Health Services | Diagnostic Centre |
| Land Use/Landcover | Health Services | Clinic |
| Land Use/Landcover | Health Services | Dispensary |
| Land Use/Landcover | Health Services | Nursing Home |
| Land Use/Landcover | Health Services | Maternity Home |
| Land Use/Landcover | Health Services | Primary Health Centre |
| Land Use/Landcover | Health Services | Community Health Centre |
| Land Use/Landcover | Health Services | Govt.Health Centre |
| Land Use/Landcover | Health Services | Urban Health Centre governed by ULB/Govt. |
| Land Use/Landcover | Health Services | Veterinary Hospital |
| Land Use/Landcover | Health Services | Veterinary Clinic |
| Land Use/Landcover | Health Services | Earlier Sanctioned Layout |
| Land Use/Landcover | Central Govt. Property | Office |
| Land Use/Landcover | Central Govt. Property | Quarter |
| Land Use/Landcover | State Govt. Property | Office |
| Land Use/Landcover | State Govt. Property | Quarter |
| Land Use/Landcover | Railway | Railway Property |
| Land Use/Landcover | Public & Semi-Public | Credit Society |
| Land Use/Landcover | Public & Semi-Public | Foreign Establishment |
| Land Use/Landcover | Public & Semi-Public | Police Station |
| Land Use/Landcover | Public & Semi-Public | Police Chowky |
| Land Use/Landcover | Public & Semi-Public | Cantonment |
| Land Use/Landcover | Public & Semi-Public | Battalion |
| Land Use/Landcover | Public & Semi-Public | Jail |
| Land Use/Landcover | Public & Semi-Public | Guesthouse |
| Land Use/Landcover | Public & Semi-Public | Community hall |
| Land Use/Landcover | Public & Semi-Public | Dharmashala |
| Land Use/Landcover | Public & Semi-Public | Auditorium |
| Land Use/Landcover | Public & Semi-Public | Town Hall |
| Land Use/Landcover | Public & Semi-Public | Multipurpose Hall |
| Land Use/Landcover | Public & Semi-Public | Drama Theater |
| Land Use/Landcover | Public & Semi-Public | Convention Centre |
| Land Use/Landcover | Public & Semi-Public | Museum |
| Land Use/Landcover | Public & Semi-Public | Public Library |
| Land Use/Landcover | Public & Semi-Public | Art Gallery & Cultural Centre |
| Land Use/Landcover | Public & Semi-Public | LPG Gas Booking Office |
| Land Use/Landcover | Public & Semi-Public | CNG Gas Booking Office |
| Land Use/Landcover | Public & Semi-Public | Ticket Booking & Reservation Office |
| Land Use/Landcover | Public & Semi-Public | Stock Exchange |
| Land Use/Landcover | Public & Semi-Public | Disaster Management Centre |
| Land Use/Landcover | Public & Semi-Public | Metrological Station |
| Land Use/Landcover | Public & Semi-Public | Dhobi Ghat |
| Land Use/Landcover | Public & Semi-Public | Crech |
| Land Use/Landcover | Public & Semi-Public | Day Care |
| Land Use/Landcover | Public & Semi-Public | Community Toilet |
| Land Use/Landcover | Public & Semi-Public | Public Toilet |
| Land Use/Landcover | Public & Semi-Public | Social Welfare Centre |
| Land Use/Landcover | Public & Semi-Public | Orphanage |
| Land Use/Landcover | Public & Semi-Public | Old Age Home |
| Land Use/Landcover | Public & Semi-Public | Night Shelter |
| Land Use/Landcover | Public & Semi-Public | Fire Station |
| Land Use/Landcover | Public & Semi-Public | Earlier Sanctioned Layout |
| Land Use/Landcover | Religious | Temple |
| Land Use/Landcover | Religious | Mosque |
| Land Use/Landcover | Religious | Idgah |
| Land Use/Landcover | Religious | Church |
| Land Use/Landcover | Religious | Gurudwara |
| Land Use/Landcover | Religious | Monastery |
| Land Use/Landcover | Religious | Synagogue |
| Land Use/Landcover | Religious | Chhatri |
| Land Use/Landcover | Religious | Agiary |
| Land Use/Landcover | Recreational | Garden |
| Land Use/Landcover | Recreational | Park |
| Land Use/Landcover | Recreational | Play Ground |
| Land Use/Landcover | Recreational | Club |
| Land Use/Landcover | Recreational | Sports Center |
| Land Use/Landcover | Recreational | Gymnasium |
| Land Use/Landcover | Recreational | Swimming Pool |
| Land Use/Landcover | Recreational | Stadium |
| Land Use/Landcover | Recreational | Planetarium |
| Land Use/Landcover | Recreational | Aquarium |
| Land Use/Landcover | Recreational | Open Air Theater |
| Land Use/Landcover | Recreational | Golf Course |
| Land Use/Landcover | Recreational | Race Course |
| Land Use/Landcover | Recreational | Exhibition Ground |
| Land Use/Landcover | Recreational | Amusement |
| Land Use/Landcover | Recreational | Theme Park |
| Land Use/Landcover | Public Utilities | Water Treatment Plant |
| Land Use/Landcover | Public Utilities | Water Pumping Station |
| Land Use/Landcover | Public Utilities | Ground Level Reservoir |
| Land Use/Landcover | Public Utilities | Sewage Treatment Plant |
| Land Use/Landcover | Public Utilities | Sewage Pumping Station |
| Land Use/Landcover | Public Utilities | Electric Power Plant |
| Land Use/Landcover | Public Utilities | Electric Sub-Station |
| Land Use/Landcover | Public Utilities | City Gate Metering Stations |
| Land Use/Landcover | Public Utilities | Area Regulator Stations |
| Land Use/Landcover | Public Utilities | Rain Water Harvesting System |
| Land Use/Landcover | Public Utilities | Effluent Treatment Plant |
| Land Use/Landcover | Public Utilities | Crematorium |
| Land Use/Landcover | Public Utilities | Burial Ground |
| Land Use/Landcover | Public Utilities | Grave Yard |
| Land Use/Landcover | Solid Waste Management | Land Fill Site |
| Land Use/Landcover | Solid Waste Management | Dumping Yard |
| Land Use/Landcover | Solid Waste Management | Recycling Plant |
| Land Use/Landcover | Communication | Telephone Exchange |
| Land Use/Landcover | Communication | Post Office |
| Land Use/Landcover | Communication | Telegraph Office |
| Land Use/Landcover | Communication | Radio Station |
| Land Use/Landcover | Communication | TV Station |
| Land Use/Landcover | Communication | Satellite & Telecommunication Centre |
| Land Use/Landcover | Heritage | Monument |
| Land Use/Landcover | Heritage | Fort |
| Land Use/Landcover | Heritage | Archaeological Site |
| Land Use/Landcover | Heritage | Heritage Precinct listed under DCPR |
| Land Use/Landcover | Heritage | Heritage Building listed under DCPR |
| Land Use/Landcover | Vacant Land | Private Vacant |
| Land Use/Landcover | Vacant Land | Municipal Asset |
| Land Use/Landcover | Vacant Land | Municipal Vacant |
| Land Use/Landcover | Vacant Land | Government Asset |
| Land Use/Landcover | Vacant Land | Government Vacant |
| Land Use/Landcover | Vacant Land | Reclaimed Land |
| Land Use/Landcover | Vacant Land | Layout |
| Land Use/Landcover | Vacant Land | Plotted Vacant Land |
| Land Use/Landcover | Transportation | Bus Stand |
| Land Use/Landcover | Transportation | Bus Terminus |
| Land Use/Landcover | Transportation | Railway Station |
| Land Use/Landcover | Transportation | Railway Yard |
| Land Use/Landcover | Transportation | Railway Siding |
| Land Use/Landcover | Transportation | Railway Track Area |
| Land Use/Landcover | Transportation | Airport |
| Land Use/Landcover | Transportation | Airstrip |
| Land Use/Landcover | Transportation | Helipad |
| Land Use/Landcover | Transportation | Port |
| Land Use/Landcover | Transportation | Harbour |
| Land Use/Landcover | Transportation | Jetty |
| Land Use/Landcover | Transportation | Truck Terminus |
| Land Use/Landcover | Transportation | Freight Complex |
| Land Use/Landcover | Transportation | Taxi Stand |
| Land Use/Landcover | Transportation | Auto Stand |
| Land Use/Landcover | Transportation | Cycle rickshaw |
| Land Use/Landcover | Transportation | Cycle |
| Land Use/Landcover | Transportation | Cart stand |
| Land Use/Landcover | Transportation | Bus Bay |
| Land Use/Landcover | Transportation | Bus Stop |
| Land Use/Landcover | Transportation | Transport Nagar |
| Land Use/Landcover | Transportation | Transport Hub |
| Land Use/Landcover | Traffic Related | Traffic Island |
| Land Use/Landcover | Traffic Related | Median |
| Land Use/Landcover | Traffic Related | Divider |
| Land Use/Landcover | Traffic Related | Parking Space |
| Land Use/Landcover | Traffic Related | Parking Area |
| Land Use/Landcover | Green Areas | Reserved Forest |
| Land Use/Landcover | Green Areas | Protected Forest |
| Land Use/Landcover | Green Areas | Social Forest |
| Land Use/Landcover | Green Areas | Green belt |
| Land Use/Landcover | Agricultural Land / Zone | Cropland |
| Land Use/Landcover | Agricultural Land / Zone | Barren land |
| Land Use/Landcover | Agricultural Land / Zone | Pad land |
| Land Use/Landcover | Agricultural Land / Zone | Plantations |
| Land Use/Landcover | Agricultural Land / Zone | Orchard |
| Land Use/Landcover | Agricultural Land / Zone | Horticulture |
| Land Use/Landcover | Agricultural Land / Zone | Plant nursery |
| Land Use/Landcover | Wetlands | Waterlogged |
| Land Use/Landcover | Wetlands | Low Lying Area |
| Land Use/Landcover | Wetlands | Marshy |
| Land Use/Landcover | Wetlands | Swampy |
| Land Use/Landcover | Wetlands | Mudflat |
| Land Use/Landcover | Wetlands | Creek |
| Land Use/Landcover | Wastelands | Scrubland |
| Land Use/Landcover | Wastelands | Barren |
| Land Use/Landcover | Wastelands | Rocky |
| Land Use/Landcover | Wastelands | Sandy Area |
| Land Use/Landcover | Wastelands | Salt Affected |
| Land Use/Landcover | Wastelands | Gullied |
| Land Use/Landcover | Specific Land Use | Mountain |
| Land Use/Landcover | Specific Land Use | Snow covered area |
| Land Use/Landcover | Specific Land Use | Mining Area |
| Land Use/Landcover | Specific Land Use | Grazing Land |
| Land Use/Landcover | Specific Land Use | Pastures |
| Land Use/Landcover | Specific Land Use | Gairan |
| Land Use/Landcover | Specific Land Use | Tea Garden |
| Land Use/Landcover | Specific Land Use | Coffee Garden |
| Land Use/Landcover | Specific Land Use | Ghats (Adjoing Water body etc.) |
| Land Use/Landcover | Specific Land Use | Sand Dunes |
| Land Use/Landcover | Eco-Sensitive Areas | Bird Sanctuary |
| Land Use/Landcover | Eco-Sensitive Areas | Bio-diversity Park |
| Land Use/Landcover | Eco-Sensitive Areas | Botanical Garden |
| Land Use/Landcover | Eco-Sensitive Areas | Zoo |
| Land Use/Landcover | Eco-Sensitive Areas | National Park |
| Land Use/Landcover | Eco-Sensitive Areas | Mangrove |
| Land Use/Landcover | Eco-Sensitive Areas | Oxbow Lakes |
| Land Use/Landcover | Eco-Sensitive Areas | Paleo channels |
| Land Use/Landcover | Others | Salt pan |
| Land Use/Landcover | Others | Aquaculture |
| Land Use/Landcover | Others | Brick kiln |
| Land Use/Landcover | Others | Quarry |
| Land Use/Landcover | Others | Aqueduct |
| Land Use/Landcover | Others | Farm house |
| Land Use/Landcover | Others | Dairy farm |
| Land Use/Landcover | Others | Poultry farm |
| Land Use/Landcover | Others | Nursery |
| Land Use/Landcover | Others | Slaughter House |
| Land Use/Landcover | Others | Dairy Booth |
| Land Use/Landcover | Others | Lighthouse |
| Land Use/Landcover | Others | Beach |
| Cell Tower, Wi-Fi Hotspot & Public Telephone Booth | Cell Tower, Wi-Fi Hotspot & Public Telephone Booth | Public Telephone Booth |
| Cell Tower, Wi-Fi Hotspot & Public Telephone Booth | Cell Tower, Wi-Fi Hotspot & Public Telephone Booth | WiFi Hotspot |
| Community Toilet | Community Toilet | Community Toilet |
| Fire Station | Fire Station | Fire Station |
| Garbage Collection Points Dumper | Garbage Collection Points Dumper | Garbage Collection Points Dumper |
| Landfill Sites and Dumping Yard | Landfill Sites and Dumping Yard | Landfill Sites and Dumping Yard |
| Slums | Slums | Slums |
| Bus Stop | Bus Stop | Bus Stop |
| ATM Meteorological Station Dairy Booth Light House and Other | Residential / Commercial Zone | ATM |
| ATM Meteorological Station Dairy Booth Light House and Other | Public & Semi-Public | Meteorological Station |
| ATM Meteorological Station Dairy Booth Light House and Other | Others | Light House |
| ATM Meteorological Station Dairy Booth Light House and Other | Others | Dairy Booth |
| Building | Residential | Township (ITP) |
| Building | Residential | Public Housing scheme |
| Building | Residential/Commercial | Retail |
| Building | Residential/Commercial | Wholesale |
| Building | Residential/Commercial | General Business |
| Building | Residential/Commercial | Hotel |
| Building | Residential/Commercial | Lodge |
| Building | Residential/Commercial | Restaurant |
| Building | Residential/Commercial | Shopping Mall |
| Building | Residential/Commercial | Shopping Center |
| Building | Residential/Commercial | Multiplex |
| Building | Residential/Commercial | Cinema |
| Building | Residential/Commercial | Functional Hall |
| Building | Residential/Commercial | Marriage Hall |
| Building | Residential/Commercial | Resort |
| Building | Residential/Commercial | Petrol Pump |
| Building | Residential/Commercial | LPG filling station |
| Building | Residential/Commercial | Informal Shop |
| Building | Residential/Commercial | Hostel |
| Building | Residential/Commercial | Private Office |
| Building | Residential/Commercial | Banks |
| Building | Residential/Commercial | Tourist Facility Center |
| Building | Industrial | Manufacturing |
| Building | Industrial | Service |
| Building | Industrial | Chemical |
| Building | Industrial | Pharmaceutical |
| Building | Industrial | Textile |
| Building | Industrial | IT Parks |
| Building | Industrial | Industrial Estate |
| Building | Industrial | SEZ |
| Building | Industrial | Agro based & Food Processing |
| Building | Industrial | Obnoxious |
| Building | Industrial | Cottage & Household |
| Building | Industrial | Other Industries |
| Building | Industrial | Warehouse |
| Building | Industrial | Storage Godown |
| Building | Mixed | Residential & Commercial |
| Building | Mixed | Residential & Household Industry |
| Building | Mixed | Residential & Educational |
| Building | Mixed | Residential & Health Services |
| Building | Mixed | Commercial & Industrial |
| Building | Mixed | Commercial & Health Services |
| Building | Mixed | Commercial & Educational |
| Building | Mixed | Commercial & Recreational |
| Building | Mixed | Residential & Commercial & Institutional |
| Building | Educational | School |
| Building | Educational | College |
| Building | Educational | University |
| Building | Educational | Vocational Institute |
| Building | Educational | ITI |
| Building | Educational | Anganwadi |
| Building | Educational | Training Institute |
| Building | Health Services | Govt. Hospital |
| Building | Health Services | Rural Hospital |
| Building | Health Services | Municipal Hospital |
| Building | Health Services | Private Hospital |
| Building | Health Services | Diagnostic Centre |
| Building | Health Services | Clinic |
| Building | Health Services | Dispensary |
| Building | Health Services | Nursing Home |
| Building | Health Services | Maternity Home |
| Building | Health Services | Primary Health Centre |
| Building | Health Services | Community Health Centre |
| Building | Health Services | Urban Health Centre Governed by ULB |
| Building | Health Services | Govt. Health Centre |
| Building | Health Services | Veterinary Hospital |
| Building | Health Services | Veterinary Clinic |
| Building | Central Govt. | Office |
| Building | Central Govt. | Quarter |
| Building | State Govt. Property | Office |
| Building | State Govt. Property | Quarter |
| Building | Railway | Railway Property |
| Building | Public & Semi-Public | Credit Society |
| Building | Public & Semi-Public | Foreign Establishment |
| Building | Public & Semi-Public | Police Station |
| Building | Public & Semi-Public | Police Chowky |
| Building | Public & Semi-Public | Cantonment |
| Building | Public & Semi-Public | Battalion |
| Building | Public & Semi-Public | Jail |
| Building | Public & Semi-Public | Guesthouse |
| Building | Public & Semi-Public | Community Hall |
| Building | Public & Semi-Public | Dharmashala |
| Building | Public & Semi-Public | Auditorium |
| Building | Public & Semi-Public | Town Hall |
| Building | Public & Semi-Public | Multipurpose Hall |
| Building | Public & Semi-Public | Drama Theater |
| Building | Public & Semi-Public | Convention Centre |
| Building | Public & Semi-Public | Museum |
| Building | Public & Semi-Public | Public Library |
| Building | Public & Semi-Public | Art Gallery & Cultural Centre |
| Building | Public & Semi-Public | LPG Gas Booking Office |
| Building | Public & Semi-Public | CNG Gas Booking Office |
| Building | Public & Semi-Public | Ticket Booking & Reservation Office |
| Building | Public & Semi-Public | Stock Exchange |
| Building | Public & Semi-Public | Disaster Management Centre |
| Building | Public & Semi-Public | Day Care |
| Building | Public & Semi-Public | Crèche |
| Building | Public & Semi-Public | Public Toilet |
| Building | Public & Semi-Public | Community Toilet |
| Building | Public & Semi-Public | Social Welfare Centre |
| Building | Public & Semi-Public | Orphanage |
| Building | Public & Semi-Public | Old Age Home |
| Building | Public & Semi-Public | Night Shelter |
| Building | Public & Semi-Public | Fire Station |
| Building | Religious | Temple |
| Building | Religious | Mosque |
| Building | Religious | Idgah |
| Building | Religious | Church |
| Building | Religious | Gurudwara |
| Building | Religious | Monastery |
| Building | Religious | Synagogue |
| Building | Religious | Chhatri |
| Building | Religious | Agiary |
| Building | Recreational | Garden |
| Building | Recreational | Park |
| Building | Recreational | Club |
| Building | Recreational | Sports Centre |
| Building | Recreational | Gymnasium |
| Building | Recreational | Swimming Pool |
| Building | Recreational | Stadium |
| Building | Recreational | Planetarium |
| Building | Recreational | Aquarium |
| Building | Recreational | open Air Theatre |
| Building | Recreational | Golf Course |
| Building | Recreational | Race Course |
| Building | Recreational | Exhibition Ground |
| Building | Recreational | Theme Park |
| Building | Recreational | Amusement |
| Building | Public Utilities | Water Treatment Plant |
| Building | Public Utilities | Water Pumping Station |
| Building | Public Utilities | Ground Level Reservoir |
| Building | Public Utilities | Sewage Treatment Plant |
| Building | Public Utilities | Sewage Pumping Station |
| Building | Public Utilities | Electric Power Plant |
| Building | Public Utilities | Electric Sub Station |
| Building | Public Utilities | Effluent Treatment Plant |
| Building | Public Utilities | Crematorium |
| Building | Public Utilities | Burial Ground |
| Building | Public Utilities | Grave Yard |
| Building | Solid Waste Management | Recycling Plant |
| Building | Communication | Telephone Exchange |
| Building | Communication | Post Office |
| Building | Communication | Telegraph Office |
| Building | Communication | Radio Station |
| Building | Communication | TV Station |
| Building | Communication | Satellite & Telecommunication Centre |
| Building | Heritage | Monument |
| Building | Heritage | Fort |
| Building | Heritage | Archaeological Site |
| Building | Transportation | Bus Stand |
| Building | Transportation | Bus Terminus |
| Building | Transportation | Railway Station |
| Building | Transportation | Railway Yard |
| Building | Transportation | Railway Sliding |
| Building | Transportation | Airport |
| Building | Transportation | Airstrip |
| Building | Transportation | Port |
| Building | Transportation | Harbour |
| Building | Transportation | Truck Terminus |
| Building | Transportation | Freight Complex |
| Building | Transportation | Transport Nagar |
| Building | Traffic Related | Multi-Level Parking |
| Building | Specific Land Use | Tea Garden |
| Building | Specific Land Use | Coffee Garden |
| Building | Eco-Sensitive Areas | Bird Sanctuary |
| Building | Eco-Sensitive Areas | Bio-diversity Park |
| Building | Eco-Sensitive Areas | Botanical Garden |
| Building | Eco-Sensitive Areas | Zoo |
| Building | Eco-Sensitive Areas | National Park |
| Building | Others | Farm House |
| Building | Others | Dairy farm |
| Building | Others | Poultry farm |
| Building | Others | Slaughter House |
| Building | Others | Dairy Booth |
| Water Supply Network Line | Water Supply Network Line | Raw Water Main Pipeline |
| Water Supply Network Line | Water Supply Network Line | Pumping Line |
| Water Supply Network Line | Water Supply Network Line | Distribution Pipeline |
| Water Supply Network Line | Water Supply Network Line | Service Pipeline |
| Water Supply Network Points | Water Supply Network Points | Water Treatment Plant |
| Water Supply Network Points | Water Supply Network Points | Water Pumping Station |
| Water Supply Network Points | Water Supply Network Points | Ground Level Reservoir |
| Water Supply Network Points | Water Supply Network Points | Supply Valve |
| Water Supply Network Points | Water Supply Network Points | Over Head Tank |
| Water Supply Network Points | Water Supply Network Points | Water tank |
| Water Supply Network Points | Water Supply Network Points | Public Stand Post |
| Water Supply Network Points | Water Supply Network Points | Tube Well |
| Water Supply Network Points | Water Supply Network Points | Hand Pump |
| Water Supply Network Points | Water Supply Network Points | Well |
| Water Supply Network Points | Water Supply Network Points | Bore Well |
| Storm Water Drainage Network Line | Storm Water Drainage Network Line | Storm Water Drain |
| Storm Water Drainage Network Points | Storm Water Drainage Network Points | Storm Water Vent |
| Sewerage Network Line | Sewerage Network Line | Pumping Line |
| Sewerage Network Line | Sewerage Network Line | Main Sewer Line |
| Sewerage Network Line | Sewerage Network Line | Branch Sewer Line |
| Sewerage Network Line | Sewerage Network Line | Service Sewer Line |
| Sewerage Network Points | Sewerage Network Points | Sewage Treatment Plant |
| Sewerage Network Points | Sewerage Network Points | Sewage Pumping Station |
| Sewerage Network Points | Sewerage Network Points | Manhole |
| Sewerage Network Points | Sewerage Network Points | Vent Valve |
| Power Supply Network Line | Power Supply Network Line | 33 Kv Line / High Tension Line |
| Power Supply Network Line | Power Supply Network Line | 11 Kv Line / Low Tension line |
| Power Supply Network Points | Power Supply Network Points | Electric Power Plant |
| Power Supply Network Points | Power Supply Network Points | Electric Sub-Station |
| Power Supply Network Points | Power Supply Network Points | Transmission Tower |
| Power Supply Network Points | Power Supply Network Points | Transformer |
| Power Supply Network Points | Power Supply Network Points | Pole |
| Power Supply Network Points | Power Supply Network Points | Street Light |
| Power Supply Network Points | Power Supply Network Points | Distribution Point |
| Power Supply Network Points | Power Supply Network Points | Electrical Box |
| Power Supply Network Points | Power Supply Network Points | Electrical Switch |
| Gas Distribution Network Line | Gas Distribution Network Line | Main Distribution Line |
| Gas Distribution Network Line | Gas Distribution Network Line | Branch Distribution Line |
| Gas Distribution Network Points | Gas Distribution Network Points | City Gate Metering Station |
| Gas Distribution Network Points | Gas Distribution Network Points | Area Regulator Station |
| Gas Distribution Network Points | Gas Distribution Network Points | Regulator |
| Gas Distribution Network Points | Gas Distribution Network Points | Flow Meter |
| Contour | Contour | Contour |
| Ground Control Point | Ground Control Point | Reference Station |
| Ground Control Point | Ground Control Point | Rover Station |
| Cadaster | Cadaster | Cadaster |
| Administrative Boundary | Administrative Boundary | International Boundary |
| Administrative Boundary | Administrative Boundary | State Boundary |
| Administrative Boundary | Administrative Boundary | District Boundary |
| Administrative Boundary | Administrative Boundary | Tehsil Boundary |
| Administrative Boundary | Administrative Boundary | Village Boundary |
| Administrative Boundary | Administrative Boundary | Forest Establishment Boundary |
| Administrative Boundary | Administrative Boundary | Defense Establishment Boundary |
| Planning Boundaries | Planning Boundaries | Planning Area Boundary |
| Planning Boundaries | Planning Boundaries | DP Boundary |
| Planning Boundaries | Planning Boundaries | Highway Corridor Development Zone |
| Planning Boundaries | Planning Boundaries | Industrial Zone |
| Planning Boundaries | Planning Boundaries | Industrial Area |
| Planning Boundaries | Planning Boundaries | Special Economic Zone |
| Planning Boundaries | Planning Boundaries | National Park |
| Planning Boundaries | Planning Boundaries | Sanctuary |
| Planning Boundaries | Planning Boundaries | Conservation Area |
| Planning Boundaries | Planning Boundaries | TPS Boundary |
| Planning Boundaries | Planning Boundaries | Planning Sector Boundary |
| Municipal Boundaries | Municipal Boundaries | Municipal Boundary |
| Municipal Boundaries | Municipal Boundaries | Planning Authority Boundary |
| Municipal Boundaries | Municipal Boundaries | Zone Boundary |
| Municipal Boundaries | Municipal Boundaries | Municipal Ward Boundary |
| Municipal Boundaries | Municipal Boundaries | Property Tax zone Boundary |
| Municipal Boundaries | Municipal Boundaries | Municipal Extended Area Boundary |
| Municipal Boundaries | Municipal Boundaries | Municipal Limit Old |
| Municipal Boundaries | Municipal Boundaries | Congested |
| Municipal Boundaries | Municipal Boundaries | Core |
| Municipal Boundaries | Municipal Boundaries | Gaothan Area Boundary |
| Municipal Boundaries | Municipal Boundaries | Notified Area Boundary |
| Other Boundaries | Other Boundaries | Mining Area Boundary |
| Other Boundaries | Other Boundaries | Eco Sensitive Zone Boundary |
| Other Boundaries | Other Boundaries | Plot Boundary |
| Other Boundaries | Other Boundaries | Census Ward Boundary |
| Other Boundaries | Other Boundaries | CRZ |
| Other Boundaries | Other Boundaries | High Tide Line |
| Other Boundaries | Other Boundaries | High Tide Line |
| Hazard Prone Areas | Hazard Prone Areas | Flood |
| Hazard Prone Areas | Hazard Prone Areas | Earthquake |
| Hazard Prone Areas | Hazard Prone Areas | Landslide |
| Religious | Religious | Dargah |
| Recreational | Recreational | Layout RG |
| Recreational | Recreational | Layout Amenity |
| Eco Sensitive Areas | Eco Sensitive Areas | National Park buffer |
| Eco Sensitive Areas | Eco Sensitive Areas | Flamingo boundary |
| PSP Private | PSP Private | Library |
| PSP Private | PSP Private | Reading room |
| PSP Private | PSP Private | Gymnasium |
| PSP Private | PSP Private | Swimming pool |
| PSP Private | PSP Private | Club house |
| PSP Private | PSP Private | Temple/ Religious Activity |
| Municipal Property | Municipal Property | Office |
| Municipal Property | Municipal Property | Quarters |
| Municipal Property | Municipal Property | Others |
| PSP Private | PSP Private | Library |
| PSP Private | PSP Private | Reading room |
| PSP Private | PSP Private | Gymnasium |
| PSP Private | PSP Private | Club house |
| PSP Private | PSP Private | Temple/ Religious Activity |
| CRZ I | CRZ I | Nesting grounds of birds |
| CRZ I | CRZ I | Turtle Nesting grounds |
| CRZ I | CRZ I | 50 m Mangrove Buffer Zone |
| CRZ I | CRZ I | Marine Protected area |
| CRZ I | CRZ I | Archaeological and Heritage sites |
| CRZ I | CRZ I | Corals and Coral Reefs |
| CRZ I | CRZ I | Reserve Forests |
| CRZ I | CRZ I | Salt Marsh |
| CRZ I | CRZ I | Sand dunes |
| CRZ I | CRZ I | Mangroves |
| CRZ I | CRZ I | Mudflats |
| CRZ I | CRZ I | Beach |
| CRZ I | CRZ I | Rock Outcrop |
| CRZ I | CRZ I | Intertidal Zone |
| CRZ II | CRZ II | CRZ Landward of HTL |
| CRZ III | CRZ III | 50-500m from HTL |
| CRZ III | CRZ III | 200-500m from HTL |
| CRZ III | CRZ III | No Development Zone |
| CRZ IV | CRZ IV | Waterbody |
| CRZ IV | CRZ IV | Waterbody |

## Annexure-B: List of Layers to be included in the LULC Map

| Layer Name | Class | Sub Class |
| --- | --- | --- |
| Land Use / Landcover | Residential Zone | Residential Area |
|  | Slum | Notified Slum |
|  | Slum | Non-Notified Slum |
|  | Vacant Land | Vacant Land |
|  | Green Areas | Notified Forest |
|  | Green Areas | Tree Clad Area |
|  | Agricultural Land / Zone | Fallow land |
|  | Specific Land Use | Hill |
|  | Others | Dam |
|  | Others | Barrage |
|  | Residential Zone | Township (ITP) |
|  | Residential Zone | Public Housing Scheme |
|  | Residential Zone | Earlier Sanctioned Layout |
|  | Residential Commercial Zone | Retail |
|  | Residential Commercial Zone | Wholesale |
|  | Residential Commercial Zone | General Business |
|  | Residential Commercial Zone | Hotel |
|  | Residential Commercial Zone | Lodge |
|  | Residential Commercial Zone | Restaurant |
|  | Residential Commercial Zone | Shopping Centre |
|  | Residential Commercial Zone | Shopping Mall |
|  | Residential Commercial Zone | Multiplex |
|  | Residential Commercial Zone | Cinema |
|  | Residential Commercial Zone | Function Hall |
|  | Residential Commercial Zone | Marriage Garden |
|  | Residential Commercial Zone | Resort |
|  | Residential Commercial Zone | Petrol Pump |
|  | Residential Commercial Zone | LPG filling station |
|  | Residential Commercial Zone | Informal Shop |
|  | Residential Commercial Zone | Hostel |
|  | Residential Commercial Zone | Market (Daily & Weekly) |
|  | Residential Commercial Zone | Mandi |
|  | Residential Commercial Zone | Private Office |
|  | Residential Commercial Zone | Banks |
|  | Residential Commercial Zone | Tourist Facility Centre |
|  | Residential Commercial Zone | Earlier Sanctioned Layout |
|  | Industrial | Manufacturing |
|  | Industrial | Service |
|  | Industrial | Chemical |
|  | Industrial | Pharmaceutical |
|  | Industrial | Textile |
|  | Industrial | IT Parks |
|  | Industrial | Industrial Estate |
|  | Industrial | SEZ |
|  | Industrial | Agro Based & Food Processing |
|  | Industrial | Obnoxious |
|  | Industrial | Cottage and Household |
|  | Industrial | Other Industries |
|  | Industrial | Warehouse |
|  | Industrial | Storage Godown |
|  | Industrial | Earlier Sanctioned Layout |
|  | Mixed | Residential & Commercial |
|  | Mixed | Residential & Household Industry |
|  | Mixed | Residential & Educational |
|  | Mixed | Residential & Health Services |
|  | Mixed | Commercial & Industrial |
|  | Mixed | Commercial & Health Services |
|  | Mixed | Commercial & Educational |
|  | Mixed | Commercial & Recreational |
|  | Mixed | Residential & Commercial & Institutional |
|  | Mixed | Residential & Handloom Industry |
|  | Mixed | Other |
|  | Mixed | Earlier Sanctioned Layout |
|  | Educational | School |
|  | Educational | College |
|  | Educational | University |
|  | Educational | Vocational Institute |
|  | Educational | ITI |
|  | Educational | Anganwari |
|  | Educational | Training Institute |
|  | Educational | Earlier Sanctioned Layout |
|  | Health Services | Govt. Hospital |
|  | Health Services | Rural Hospital |
|  | Health Services | Municipal Hospital |
|  | Health Services | Private Hospital |
|  | Health Services | Diagnostic Centre |
|  | Health Services | Clinic |
|  | Health Services | Dispensary |
|  | Health Services | Nursing Home |
|  | Health Services | Maternity Home |
|  | Health Services | Primary Health Centre |
|  | Health Services | Community Health Centre |
|  | Health Services | Govt.Health Centre |
|  | Health Services | Urban Health Centre governed by ULB/Govt. |
|  | Health Services | Veterinary Hospital |
|  | Health Services | Veterinary Clinic |
|  | Health Services | Earlier Sanctioned Layout |
|  | Central Govt. Property | Office |
|  | Central Govt. Property | Quarter |
|  | State Govt. Property | Office |
|  | State Govt. Property | Quarter |
|  | Public & Semi-Public | Credit Society |
|  | Public & Semi-Public | Foreign Establishment |
|  | Public & Semi-Public | Police Station |
|  | Public & Semi-Public | Police Chowky |
|  | Public & Semi-Public | Cantonment |
|  | Public & Semi-Public | Battalion |
|  | Public & Semi-Public | Jail |
|  | Public & Semi-Public | Guesthouse |
|  | Public & Semi-Public | Community hall |
|  | Public & Semi-Public | Dharmashala |
|  | Public & Semi-Public | Auditorium |
|  | Public & Semi-Public | Town Hall |
|  | Public & Semi-Public | Multipurpose Hall |
|  | Public & Semi-Public | Drama Theatre |
|  | Public & Semi-Public | Convention Centre |
|  | Public & Semi-Public | Museum |
|  | Public & Semi-Public | Public Library |
|  | Public & Semi-Public | Art Gallery & Cultural Centre |
|  | Public & Semi-Public | LPG Gas Booking Office |
|  | Public & Semi-Public | CNG Gas Booking Office |
|  | Public & Semi-Public | Ticket Booking & Reservation Office |
|  | Public & Semi-Public | Stock Exchange |
|  | Public & Semi-Public | Disaster Management Centre |
|  | Public & Semi-Public | Metrological Station |
|  | Public & Semi-Public | Dhobi Ghat |
|  | Public & Semi-Public | Crech |
|  | Public & Semi-Public | Day Care |
|  | Public & Semi-Public | Community Toilet |
|  | Public & Semi-Public | Public Toilet |
|  | Public & Semi-Public | Social Welfare Centre |
|  | Public & Semi-Public | Orphanage |
|  | Public & Semi-Public | Old Age Home |
|  | Public & Semi-Public | Night Shelter |
|  | Public & Semi-Public | Fire Station |
|  | Public & Semi-Public | Earlier Sanctioned Layout |
|  | Religious | Temple |
|  | Religious | Mosque |
|  | Religious | Idgah |
|  | Religious | Church |
|  | Religious | Gurudwara |
|  | Religious | Monastery |
|  | Religious | Synagogue |
|  | Religious | Chhatri |
|  | Religious | Agiary |
|  | Recreational | Garden |
|  | Recreational | Park |
|  | Recreational | Play Ground |
|  | Recreational | Club |
|  | Recreational | Sports Centre |
|  | Recreational | Gymnasium |
|  | Recreational | Swimming Pool |
|  | Recreational | Stadium |
|  | Recreational | Planetarium |
|  | Recreational | Aquarium |
|  | Recreational | Open Air Theatre |
|  | Recreational | Golf Course |
|  | Recreational | Race Course |
|  | Recreational | Exhibition Ground |
|  | Recreational | Amusement |
|  | Recreational | Theme Park |
|  | Public Utilities | Water Treatment Plant |
|  | Public Utilities | Water Pumping Station |
|  | Public Utilities | Ground Level Reservoir |
|  | Public Utilities | Sewage Treatment Plant |
|  | Public Utilities | Sewage Pumping Station |
|  | Public Utilities | Electric Power Plant |
|  | Public Utilities | Electric Sub-Station |
|  | Public Utilities | City Gate Metering Stations |
|  | Public Utilities | Area Regulator Stations |
|  | Public Utilities | Rain Water Harvesting System |
|  | Public Utilities | Effluent Treatment Plant |
|  | Public Utilities | Crematorium |
|  | Public Utilities | Burial Ground |
|  | Public Utilities | Grave Yard |
|  | Solid Waste Management | Land Fill Site |
|  | Solid Waste Management | Dumping Yard |
|  | Solid Waste Management | Recycling Plant |
|  | Communication | Telephone Exchange |
|  | Communication | Post Office |
|  | Communication | Telegraph Office |
|  | Communication | Radio Station |
|  | Communication | TV Station |
|  | Communication | Satellite & Telecommunication Centre |
|  | Heritage | Monument |
|  | Heritage | Fort |
|  | Heritage | Archaeological Site |
|  | Heritage | Heritage Precinct listed under DCPR |
|  | Heritage | Heritage Building listed under DCPR |
|  | Vacant Land | Private Vacant |
|  | Vacant Land | Municipal Asset |
|  | Vacant Land | Municipal Vacant |
|  | Vacant Land | Government Asset |
|  | Vacant Land | Government Vacant |
|  | Vacant Land | Reclaimed Land |
|  | Vacant Land | Layout |
|  | Vacant Land | Plotted Vacant Land |
|  | Transportation | Bus Stand |
|  | Transportation | Bus Terminus |
|  | Transportation | Railway Station |
|  | Transportation | Railway Yard |
|  | Transportation | Railway Siding |
|  | Transportation | Railway Track Area |
|  | Transportation | Airport |
|  | Transportation | Airstrip |
|  | Transportation | Helipad |
|  | Transportation | Port |
|  | Transportation | Harbour |
|  | Transportation | Jetty |
|  | Transportation | Truck Terminus |
|  | Transportation | Freight Complex |
|  | Transportation | Taxi Stand |
|  | Transportation | Auto Stand |
|  | Transportation | Cycle rickshaw |
|  | Transportation | Cycle |
|  | Transportation | Cart stand |
|  | Transportation | Bus Bay |
|  | Transportation | Bus Stop |
|  | Transportation | Transport Nagar |
|  | Transportation | Transport Hub |
|  | Traffic Related | Traffic Island |
|  | Traffic Related | Median |
|  | Traffic Related | Divider |
|  | Traffic Related | Parking Space |
|  | Traffic Related | Parking Area |
|  | Green Areas | Reserved Forest |
|  | Green Areas | Protected Forest |
|  | Green Areas | Social Forest |
|  | Green Areas | Green belt |
|  | Agricultural Land / Zone | Cropland |
|  | Agricultural Land / Zone | Barren land |
|  | Agricultural Land / Zone | Pad land |
|  | Agricultural Land / Zone | Plantations |
|  | Agricultural Land / Zone | Orchard |
|  | Agricultural Land / Zone | Horticulture |
|  | Agricultural Land / Zone | Plant nursery |
|  | Wetlands | Waterlogged |
|  | Wetlands | Low Lying Area |
|  | Wetlands | Marshy |
|  | Wetlands | Swampy |
|  | Wetlands | Mudflat |
|  | Wetlands | Creek |
|  | Wastelands | Scrubland |
|  | Wastelands | Barren |
|  | Wastelands | Rocky |
|  | Wastelands | Sandy Area |
|  | Wastelands | Salt Affected |
|  | Wastelands | Gullied |
|  | Specific Land Use | Mountain |
|  | Specific Land Use | Snow covered area |
|  | Specific Land Use | Mining Area |
|  | Specific Land Use | Grazing Land |
|  | Specific Land Use | Pastures |
|  | Specific Land Use | Gairan |
|  | Specific Land Use | Tea Garden |
|  | Specific Land Use | Coffee Garden |
|  | Specific Land Use | Ghats (Adjoing Water body etc.) |
|  | Specific Land Use | Sand Dunes |
|  | Eco-Sensitive Areas | Bird Sanctuary |
|  | Eco-Sensitive Areas | Bio-diversity Park |
|  | Eco-Sensitive Areas | Botanical Garden |
|  | Eco-Sensitive Areas | Zoo |
|  | Eco-Sensitive Areas | National Park |
|  | Eco-Sensitive Areas | Mangrove |
|  | Eco-Sensitive Areas | Oxbow Lakes |
|  | Eco-Sensitive Areas | Paleo channels |
|  | Others | Salt pan |
|  | Others | Aquaculture |
|  | Others | Brick kiln |
|  | Others | Quarry |
|  | Others | Aqueduct |
|  | Others | Farm house |
|  | Others | Dairy farm |
|  | Others | Poultry farm |
|  | Others | Nursery |
|  | Others | Slaughter House |
|  | Others | Dairy Booth |
|  | Others | Lighthouse |
|  | Others | Beach |
| Tree | Tree | Tree |
| Cell Tower, Wi-Fi Hotspot & Public Telephone Booth | Cell Tower, Wi-Fi Hotspot & Public Telephone Booth | Cell Tower |
|  | Cell Tower, Wi-Fi Hotspot & Public Telephone Booth | Public Telephone Booth |
|  | Cell Tower, Wi-Fi Hotspot & Public Telephone Booth | WiFi Hotspot |
| Community Toilet | Community Toilet | Community Toilet |
| Fire Station | Fire Station | Fire Station |
| Garbage Collection Points Dumper | Garbage Collection Points Dumper | Garbage Collection Points Dumper |
| Landfill Sites and Dumping Yard | Landfill Sites and Dumping Yard | Landfill Sites and Dumping Yard |
| Slums | Slums | Slums |
| Bus Stop | Bus Stop | Bus Stop |
| ATM Meteorological Station Dairy Booth Light House and Other | Residential / Commercial Zone | ATM |
|  | Public & Semi-Public | Meteorological Station |
|  | Others | Light House |
|  | Others | Dairy Booth |
| Building | Residential | Residential Area |
|  | Residential | Township (ITP) |
|  | Residential | Public Housing scheme |
|  | Residential/Commercial | Retail |
|  | Residential/Commercial | Wholesale |
|  | Residential/Commercial | General Business |
|  | Residential/Commercial | Hotel |
|  | Residential/Commercial | Lodge |
|  | Residential/Commercial | Restaurant |
|  | Residential/Commercial | Shopping Mall |
|  | Residential/Commercial | Shopping Centre |
|  | Residential/Commercial | Multiplex |
|  | Residential/Commercial | Cinema |
|  | Residential/Commercial | Functional Hall |
|  | Residential/Commercial | Marriage Hall |
|  | Residential/Commercial | Resort |
|  | Residential/Commercial | Petrol Pump |
|  | Residential/Commercial | LPG filling station |
|  | Residential/Commercial | Informal Shop |
|  | Residential/Commercial | Hostel |
|  | Residential/Commercial | Private Office |
|  | Residential/Commercial | Banks |
|  | Residential/Commercial | Tourist Facility Centre |
|  | Industrial | Manufacturing |
|  | Industrial | Service |
|  | Industrial | Chemical |
|  | Industrial | Pharmaceutical |
|  | Industrial | Textile |
|  | Industrial | IT Parks |
|  | Industrial | Industrial Estate |
|  | Industrial | SEZ |
|  | Industrial | Agro based & Food Processing |
|  | Industrial | Obnoxious |
|  | Industrial | Cottage & Household |
|  | Industrial | Other Industries |
|  | Industrial | Warehouse |
|  | Industrial | Storage Godown |
|  | Mixed | Residential & Commercial |
|  | Mixed | Residential & Household Industry |
|  | Mixed | Residential & Educational |
|  | Mixed | Residential & Health Services |
|  | Mixed | Commercial & Industrial |
|  | Mixed | Commercial & Health Services |
|  | Mixed | Commercial & Educational |
|  | Mixed | Commercial & Recreational |
|  | Mixed | Residential & Commercial & Institutional |
|  | Educational | School |
|  | Educational | College |
|  | Educational | University |
|  | Educational | Vocational Institute |
|  | Educational | ITI |
|  | Educational | Anganwadi |
|  | Educational | Training Institute |
|  | Health Services | Govt. Hospital |
|  | Health Services | Rural Hospital |
|  | Health Services | Municipal Hospital |
|  | Health Services | Private Hospital |
|  | Health Services | Diagnostic Centre |
|  | Health Services | Clinic |
|  | Health Services | Dispensary |
|  | Health Services | Nursing Home |
|  | Health Services | Maternity Home |
|  | Health Services | Primary Health Centre |
|  | Health Services | Community Health Centre |
|  | Health Services | Urban Health Centre Governed by ULB |
|  | Health Services | Govt. Health Centre |
|  | Health Services | Veterinary Hospital |
|  | Health Services | Veterinary Clinic |
|  | Central Govt. | Office |
|  | Central Govt. | Quarter |
|  | State Govt. Property | Office |
|  | State Govt. Property | Quarter |
|  | Public & Semi-Public | Credit Society |
|  | Public & Semi-Public | Foreign Establishment |
|  | Public & Semi-Public | Police Station |
|  | Public & Semi-Public | Police Chowky |
|  | Public & Semi-Public | Cantonment |
|  | Public & Semi-Public | Battalion |
|  | Public & Semi-Public | Jail |
|  | Public & Semi-Public | Guesthouse |
|  | Public & Semi-Public | Community Hall |
|  | Public & Semi-Public | Dharmashala |
|  | Public & Semi-Public | Auditorium |
|  | Public & Semi-Public | Town Hall |
|  | Public & Semi-Public | Multipurpose Hall |
|  | Public & Semi-Public | Drama Theatre |
|  | Public & Semi-Public | Convention Centre |
|  | Public & Semi-Public | Museum |
|  | Public & Semi-Public | Public Library |
|  | Public & Semi-Public | Art Gallery & Cultural Centre |
|  | Public & Semi-Public | LPG Gas Booking Office |
|  | Public & Semi-Public | CNG Gas Booking Office |
|  | Public & Semi-Public | Ticket Booking & Reservation Office |
|  | Public & Semi-Public | Stock Exchange |
|  | Public & Semi-Public | Disaster Management Centre |
|  | Public & Semi-Public | Day Care |
|  | Public & Semi-Public | Crech |
|  | Public & Semi-Public | Public Toilet |
|  | Public & Semi-Public | Community Toilet |
|  | Public & Semi-Public | Social Welfare Centre |
|  | Public & Semi-Public | Orphanage |
|  | Public & Semi-Public | Old Age Home |
|  | Public & Semi-Public | Night Shelter |
|  | Public & Semi-Public | Fire Station |
|  | Religious | Temple |
|  | Religious | Mosque |
|  | Religious | Idgah |
|  | Religious | Church |
|  | Religious | Gurudwara |
|  | Religious | Monastery |
|  | Religious | Synagogue |
|  | Religious | Chhatri |
|  | Religious | Agiary |
|  | Recreational | Garden |
|  | Recreational | Park |
|  | Recreational | Club |
|  | Recreational | Sports Centre |
|  | Recreational | Gymnasium |
|  | Recreational | Swimming Pool |
|  | Recreational | Stadium |
|  | Recreational | Planetarium |
|  | Recreational | Aquarium |
|  | Recreational | open Air Theatre |
|  | Recreational | Golf Course |
|  | Recreational | Race Course |
|  | Recreational | Exhibition Ground |
|  | Recreational | Theme Park |
|  | Recreational | Amusement |
|  | Public Utilities | Water Treatment Plant |
|  | Public Utilities | Water Pumping Station |
|  | Public Utilities | Ground Level Reservoir |
|  | Public Utilities | Sewage Treatment Plant |
|  | Public Utilities | Sewage Pumping Station |
|  | Public Utilities | Electric Power Plant |
|  | Public Utilities | Electric Sub Station |
|  | Public Utilities | Effluent Treatment Plant |
|  | Public Utilities | Crematorium |
|  | Public Utilities | Burial Ground |
|  | Public Utilities | Grave Yard |
|  | Solid Waste Management | Recycling Plant |
|  | Communication | Telephone Exchange |
|  | Communication | Post Office |
|  | Communication | Telegraph Office |
|  | Communication | Radio Station |
|  | Communication | TV Station |
|  | Communication | Satellite & Telecommunication Centre |
|  | Heritage | Monument |
|  | Heritage | Fort |
|  | Heritage | Archaeological Site |
|  | Transportation | Bus Stand |
|  | Transportation | Bus Terminus |
|  | Transportation | Railway Station |
|  | Transportation | Railway Yard |
|  | Transportation | Railway Sliding |
|  | Transportation | Airport |
|  | Transportation | Airstrip |
|  | Transportation | Port |
|  | Transportation | Harbour |
|  | Transportation | Truck Terminus |
|  | Transportation | Freight Complex |
|  | Transportation | Transport Nagar |
|  | Traffic Related | Multi-Level Parking |
|  | Specific Land Use | Tea Garden |
|  | Specific Land Use | Coffee Garden |
|  | Eco-Sensitive Areas | Bird Sanctuary |
|  | Eco-Sensitive Areas | Bio-diversity Park |
|  | Eco-Sensitive Areas | Botanical Garden |
|  | Eco-Sensitive Areas | Zoo |
|  | Eco-Sensitive Areas | National Park |
|  | Others | Farm House |
|  | Others | Dairy farm |
|  | Others | Poultry farm |
|  | Others | Slaughter House |
|  | Others | Dairy Booth |
| Water Supply Network Line | Water Supply Network Line | Raw Water Main Pipeline |
|  | Water Supply Network Line | Pumping Line |
|  | Water Supply Network Line | Distribution Pipeline |
|  | Water Supply Network Line | Service Pipeline |
| Water Supply Network Points | Water Supply Network Points | Water Treatment Plant |
|  | Water Supply Network Points | Water Pumping Station |
|  | Water Supply Network Points | Ground Level Reservoir |
|  | Water Supply Network Points | Supply Valve |
|  | Water Supply Network Points | Over Head Tank |
|  | Water Supply Network Points | Water tank |
|  | Water Supply Network Points | Public Stand Post |
|  | Water Supply Network Points | Tube Well |
|  | Water Supply Network Points | Hand Pump |
|  | Water Supply Network Points | Well |
|  | Water Supply Network Points | Bore Well |
| Storm Water Drainage Network Line | Storm Water Drainage Network Line | Storm Water Drain |
| Storm Water Drainage Network Points | Storm Water Drainage Network Points | Storm Water Vent |
| Sewerage Network Line | Sewerage Network Line | Pumping Line |
|  | Sewerage Network Line | Main Sewer Line |
|  | Sewerage Network Line | Branch Sewer Line |
|  | Sewerage Network Line | Service Sewer Line |
| Sewerage Network Points | Sewerage Network Points | Sewage Treatment Plant |
|  | Sewerage Network Points | Sewage Pumping Station |
|  | Sewerage Network Points | Manhole |
|  | Sewerage Network Points | Vent Valve |
| Power Supply Network Points | Power Supply Network Points | Electrical Box |
| Gas Distribution Network Line | Gas Distribution Network Line | Main Distribution Line |
|  | Gas Distribution Network Line | Branch Distribution Line |
| Gas Distribution Network Points | Gas Distribution Network Points | City Gate Metering Station |
|  | Gas Distribution Network Points | Area Regulator Station |
|  | Gas Distribution Network Points | Regulator |
|  | Gas Distribution Network Points | Flow Meter |
| Other Boundaries | Other Boundaries | Mining Area Boundary |
|  | Other Boundaries | Eco Sensitive Zone Boundary |
|  | Other Boundaries | Plot Boundary |
|  | Other Boundaries | Census Ward Boundary |
|  | Other Boundaries | CRZ |
|  | Other Boundaries | High Tide Line |
|  | Other Boundaries | High Tide Line |
| Hazard Prone Areas | Hazard Prone Areas | Flood |
|  | Hazard Prone Areas | Earthquake |
|  | Hazard Prone Areas | Landslide |
| Religious | Religious | Dargah |
| Recreational | Recreational | Layout RG |
|  | Recreational | Layout Amenity |
| Eco Sensitive Areas | Eco Sensitive Areas | National Park buffer |
|  | Eco Sensitive Areas | Flamingo boundary |
| Municipal Property | Municipal Property | Office |
|  | Municipal Property | Quarters |
|  | Municipal Property | Others |
| PSP Private | PSP Private | Library |
|  | PSP Private | Reading room |
|  | PSP Private | Gymnasium |
|  | PSP Private | Swimming pool |
|  | PSP Private | Club house |
|  | PSP Private | Temple/ Religious Activity |
|  | PSP Private | Library |
|  | PSP Private | Reading room |
|  | PSP Private | Gymnasium |
|  | PSP Private | Club house |
|  | PSP Private | Temple/ Religious Activity |
| CRZ I | CRZ I | Nesting grounds of birds |
|  | CRZ I | Turtle Nesting grounds |
|  | CRZ I | 50 m Mangrove Buffer Zone |
|  | CRZ I | Marine Protected area |
|  | CRZ I | Archaeological and Heritage sites |
|  | CRZ I | Corals and Coral Reefs |
|  | CRZ I | Reserve Forests |
|  | CRZ I | Salt Marsh |
|  | CRZ I | Sand dunes |
|  | CRZ I | Mangroves |
|  | CRZ I | Mudflats |
|  | CRZ I | Beach |
|  | CRZ I | Rock Outcrop |
|  | CRZ I | Intertidal Zone |
| CRZ II | CRZ II | CRZ Landward of HTL |
| CRZ III | CRZ III | 50-500m from HTL |
|  | CRZ III | 200-500m from HTL |
|  | CRZ III | No Development Zone |
| CRZ IV | CRZ IV | Waterbody |
|  | CRZ IV | Waterbody |